Embedded System design

EGR 226

Registers_Summary_and_vectors

MSP432

DR. Nabeeh Kandalaft

February, 2018

Revision I

REGISTER DESCRIPTION

Interrupts vectors

&

Registers

Interrupt vectors

startup_msp432p401r_ccs.c

Reset_Handler,                         			// the reset handler

NMI_Handler,                           			// The NMI handler

HardFault_Handler,                  			// the hard fault handler

MemManage_Handler,                     			// the MPU fault handler

BusFault_Handler,                      			// the bus fault handler

UsageFault_Handler,				// the usage fault handler

0,                                     				// Reserved

0 ,							// Reserved

0,                                    				 // Reserved

0,                                    				 // Reserved

SVC_Handler,                          			 // SVCall handler

DebugMon_Handler,                     			 // Debug monitor handler

0,                                    				 // Reserved

PendSV_Handler,                        			// The PendSV handler

SysTick_Handler,                       			// The SysTick handler

PSS_IRQHandler,                        			// PSS Interrupt

CS_IRQHandler,                        			// CS Interrupt

PCM_IRQHandler,                        			// PCM Interrupt

WDT_A_IRQHandler,                      			// WDT_A Interrupt

FPU_IRQHandler,                        			// FPU Interrupt

FLCTL_IRQHandler,                     			// Flash Controller Interrupt

COMP_E0_IRQHandler,                   		// COMP_E0 Interrupt

COMP_E1_IRQHandler,                   		// COMP_E1 Interrupt

TA0_0_IRQHandler,                     			 // TA0_0 Interrupt

TA0_N_IRQHandler,                    			 // TA0_N Interrupt

TA1_0_IRQHandler,                      			// TA1_0 Interrupt

TA1_N_IRQHandler,                     			// TA1_N Interrupt

TA2_0_IRQHandler,                     			// TA2_0 Interrupt

TA2_N_IRQHandler,                      			// TA2_N Interrupt

TA3_0_IRQHandler,                      			// TA3_0 Interrupt

TA3_N_IRQHandler,                     			 // TA3_N Interrupt

EUSCIA0_IRQHandler,                   			 // EUSCIA0 Interrupt

EUSCIA1_IRQHandler,                   			 // EUSCIA1 Interrupt

EUSCIA2_IRQHandler,                    		// EUSCIA2 Interrupt

EUSCIA3_IRQHandler,                   			 // EUSCIA3 Interrupt

EUSCIB0_IRQHandler,                    		// EUSCIB0 Interrupt

EUSCIB1_IRQHandler,                   			 // EUSCIB1 Interrupt

EUSCIB2_IRQHandler,                   			 // EUSCIB2 Interrupt

EUSCIB3_IRQHandler,                   			// EUSCIB3 Interrupt

ADC14_IRQHandler,                      			// ADC14 Interrupt

T32_INT1_IRQHandler,                  			 // T32_INT1 Interrupt

T32_INT2_IRQHandler,                  			 // T32_INT2 Interrupt

T32_INTC_IRQHandler,                  			 // T32_INTC Interrupt

AES256_IRQHandler,                     			// AES256 Interrupt

RTC_C_IRQHandler,                     			// RTC_C Interrupt

DMA_ERR_IRQHandler,                   		 // DMA_ERR Interrupt

DMA_INT3_IRQHandler,                  		 // DMA_INT3 Interrupt

DMA_INT2_IRQHandler,                   		// DMA_INT2 Interrupt

DMA_INT1_IRQHandler,                   		// DMA_INT1 Interrupt

DMA_INT0_IRQHandler,                  		 // DMA_INT0 Interrupt

PORT1_IRQHandler,                     			 // Port1 Interrupt

PORT2_IRQHandler,                      			// Port2 Interrupt

PORT3_IRQHandler,                      			// Port3 Interrupt

PORT4_IRQHandler,                     			//  Port4 Interrupt

PORT5_IRQHandler,                     			 // Port5 Interrupt

PORT6_IRQHandler                      			 // Port6 Interrupt

ISER0 Register    IRQ 0 to 31 Set Enable Register.

Use the Interrupt Set-Enable Registers to enable interrupts and determine which interrupts are currently enabled.

Figure 2-20. ISER0 Register

ISER1 Register   IRQ 32 to 63 Set Enable Register.

Use the Interrupt Set-Enable Registers to enable interrupts and determine which interrupts are currently enabled.

Figure 2-21. ISER1 Register

PSS_IRQn           		 = 0,      // 16 PSS Interrupt

CS_IRQn           		 = 1,      // 17 CS Interrupt

PCM_IRQn         	 	 = 2,      // 18 PCM Interrupt

WDT_A_IRQn        	 = 3,      // 19 WDT_A Interrupt

FPU_IRQn          	 	 = 4,      // 20 FPU Interrupt

FLCTL_IRQn         	 = 5,      // 21 Flash Controller Interrupt

To enable the interrupt vectors from  0-31 use ISER0 register

NVIC->ISER[0] = 1 << ( (ADC14_IRQn) & 31);

NVIC->ISER[0] = 1 << ( (TA0_0_IRQn)  & 31);

Or

NVIC_EnableIRQ (TA0_N_IRQn);

NVIC_EnableIRQ (ADC14_IRQn);

COMP_E0_IRQn       	 = 6,      // 22 COMP_E0 Interrupt

COMP_E1_IRQn       	 = 7,      // 23 COMP_E1 Interrupt

TA0_0_IRQn         		 = 8,      // 24 TA0_0 Interrupt

TA0_N_IRQn        		 = 9,      // 25 TA0_N Interrupt

TA1_0_IRQn         		 = 10,     // 26 TA1_0 Interrupt

TA1_N_IRQn        		 = 11,     // 27 TA1_N Interrupt

TA2_0_IRQn         		 = 12,     // 28 TA2_0 Interrupt

TA2_N_IRQn        		 = 13,     // 29 TA2_N Interrupt

TA3_0_IRQn         		 = 14,     // 30 TA3_0 Interrupt

TA3_N_IRQn         		 = 15,     // 31 TA3_N Interrupt

EUSCIA0_IRQn      	 = 16,     // 32 EUSCIA0 Interrupt

EUSCIA1_IRQn     		 = 17,     // 33 EUSCIA1 Interrupt

EUSCIA2_IRQn      	 = 18,     // 34 EUSCIA2 Interrupt

To enable the interrupt vectors from 32-40 use ISER1 register

NVIC->ISER[1] = 1 << ( (PORT1_IRQn) & 31);

Or

NVIC_EnableIRQ (PORT1_IRQn);

EUSCIA3_IRQn      	 = 19,     // 35 EUSCIA3 Interrupt

EUSCIB0_IRQn      	 = 20,     // 36 EUSCIB0 Interrupt

EUSCIB1_IRQn       	 = 21,     // 37 EUSCIB1 Interrupt

EUSCIB2_IRQn       	 = 22,     // 38 EUSCIB2 Interrupt

EUSCIB3_IRQn      	 = 23,     // 39 EUSCIB3 Interrupt

ADC14_IRQn         		 = 24,     // 40 ADC14 Interrupt

T32_INT1_IRQn     	 = 25,     // 41 T32_INT1 Interrupt

T32_INT2_IRQn      	 = 26,     // 42 T32_INT2 Interrupt

// Enable all interrupts for MSP432 (that are turned on)

// declare before the while loop

__enable_interrupt ( );

T32_INTC_IRQn      	 = 27,     // 43 T32_INTC Interrupt

AES256_IRQn        		= 28,     // 44 AES256 Interrupt

RTC_C_IRQn         	= 29,     // 45 RTC_C Interrupt

DMA_ERR_IRQn     	= 30,     // 46 DMA_ERR Interrupt

DMA_INT3_IRQn     	= 31,     // 47 DMA_INT3 Interrupt

DMA_INT2_IRQn     	= 32,     // 48 DMA_INT2 Interrupt

DMA_INT1_IRQn     	= 33,     // 49 DMA_INT1 Interrupt

DMA_INT0_IRQn      	= 34,     // 50 DMA_INT0 Interrupt

PORT1_IRQn        	 	= 35,     // 51 Port1 Interrupt

PORT2_IRQn        		= 36,     // 52 Port2 Interrupt

PORT3_IRQn         		= 37,     // 53 Port3 Interrupt

PORT4_IRQn         		= 38,     // 54 Port4 Interrupt

PORT5_IRQn        		= 39,     // 55 Port5 Interrupt

PORT6_IRQn         		= 40      // 56 Port6 Interrupt

MSP432

Registers

Pin Configuration Registers

Set PORT3 pin 1 to output			P3->DIR |= BIT1;

Set PORT3 pin 1 to input	  		P3->DIR &=~ BIT1;

To check a status of pin1 on PORT3 	 if (P3->IN & BIT1)

Use the internal pull-up / pulldown resistor on pin1 PORT2          P3->REN |= BIT1;

Set pin1 PORT 3 to high (assume the pin is an output) 				P3->OUT |= BIT1;

Set pin1 PORT3 to pull-up (pin1 is an input with enable internal resistor)  	P3->OUT |= BIT1;

Set pin1 PORT3 to pulldown (pin1 is an input with enable internal resistor)  	P3->OUT &=~ BIT1;

From Table 4.1 in Valvano’s textbook Introduction to the MSP432 Microcontroller

PxSEL0 PxSEL1 Registers

Port X Function Selection Register 0 (X = 1, 2, 3, 4, 5, 6, 7, 8, 9, 10 or J)

Partial Table of Alternate Pin Functions

PxIES Register

Port X Interrupt Edge Select Register (X = 1, 2, 3, 4, 5, 6, 7, 8, 9 or 10)

Figure 10-10. PxIES Register

Example

P1->IES |= BIT1 |BIT4; 	//Set 1.1, 1.4 pins interrupt to trigger when it goes from high to low

PxIE Register

Port X Interrupt Enable Register (X = 1, 2, 3, 4, 5, 6, 7, 8, 9 or 10)

Figure 10-11. PxIE Register

Example

P1 -> IE |= BIT1 |BIT4;		// Set interrupt on for p1.1 and 1.4

PxIFG Register

Port X Interrupt Flag Register (X = 1, 2, 3, 4, 5, 6, 7, 8, 9 or 10)

Example interrupt on switch P1.1 toggle the LED on PORT1 pin 0.

If (P1->IFG & BIT1)

P1->OUT ^= BIT0;	// Toggle the LED

P1->IFG &= ~BIT1;		// always clear the flag before you exit the interrupt ISR

***********************************************************************************************

P1.1 is configured as an input. Pressing the button connected to P1.1 service an interrupt service routine ISR results in toggling an LED on PORT3 pin5 in device waking up and servicing the Port1 ISR.

MCLK = SMCLK = 3MHz

***********************************************************************************************

#include "msp.h"

int main(void)

{

WDT_A->CTL = WDT_A_CTL_PW | WDT_A_CTL_HOLD;  	// Hold the watchdog

Port1_init ( );

Port3_init ( );

NVIC->ISER[1] = 1 << ((PORT1_IRQn) & 31);   		// Enable Port 1 interrupt on the NVIC

__enable_interrupt ( );                     				// Enable global interrupt

while ( 1 )

{

;       // Hardware is doing all the work, so nothing more to do in this while loop.

}

}

void Port1_init (void);

{

P1->SEL0 &=~ BIT1;

P1->SEL1 &=~ BIT1;

P1->DIR   &=~ BIT1;

P1->REN     =   BIT1;  	// Enable pull-up resistor (P1.1 output high)

P1->OUT     =   BIT1;

P1->IES       =   BIT1;  	//Set pin interrupt to trigger when it goes from high to low

P1->IE         =   BIT1;  	// Enable interrupt for P1.1

P1->IFG      =   0;      	// Clear all P1 interrupt flags

}

void Port3_init (void)

{

P3->SEL0 &=~ BIT5;

P3->SEL1 &=~ BIT5;

P3->DIR      |=   BIT5;

}

void PORT1_IRQHandler(void) // Port1 ISR

{

If ( P1->IFG & BIT1 )		// If P1.1 had an interrupt (going from high to low

P3->OUT ^= BIT5;       		// Toggling the output on the LED

P1->IFG &= ~BIT1;			// Reset the interrupt flag

}

Example:

Sets up P1.1 to toggle Green LED on falling edge PORT3 pin0		// with interrupts

Sets up P1.4 to toggle blue    LED on falling edge PORT3 pin1		// with interrupts

#include "msp.h"

void SetupLEDs (void);

SetupPort1Interrupts (void);

void main(void)

{

WDT_A->CTL = WDT_A_CTL_PW | WDT_A_CTL_HOLD;	// stop watchdog timer

SetupLEDs ( );                     					//Sets up the LEDs as outputs

SetupPort1Interrupts ( );   					//Initializes the buttons that interrupt the program

NVIC->ISER[1] = 1 << ((PORT1_IRQn) & 31);   		// Enable Port 1 interrupt on the NVIC

__enable_interrupt ( );                           				//Enable all interrupts that are turned on

while (1)

{

;	// Hardware is doing all the work, so nothing more to do in this while loop

}

}

void PORT1_IRQHandler(void)

{

int status = P1->IFG;            	 //Record all flags of all interrupts on this port

If (status & BIT1)                	 //If P1.1 had an interrupt (comparing the status with the BIT)

P2->OUT ^= BIT1;            	 //Turn Green On

If (status & BIT4)                  	//If P1.4 had an interrupt (comparing the status with the BIT)

P2->OUT ^= BIT2;              	//Turn Blue On

P1->IFG = 0;                      	 //Clear all flags

}

void SetupLEDs(void)

{

P3->SEL0 &=~ BIT0;          	// Setup the LED2 on the Launchpad as GPIO, Output, Off

P3->SEL1 &=~ BIT0;

P3->DIR     |=    BIT0;

P3->OUT &=~  BIT0;

P3->SEL0 &=~ BIT1;          	// Setup the LED2 on the Launchpad as GPIO, Output, Off

P3->SEL1 &=~ BIT1;

P3->DIR     |=    BIT1;

P3->OUT  &=~ BIT1;

}

void SetupPort1Interrupts ( )

{

P1->SEL0 &=~ BIT1;          	// Setup the P1.1 on the Launchpad as Input, Pull Up Resistor

P1->SEL1 &=~ BIT1;

P1->DIR   &=~ BIT1;

P1->REN    |=   BIT1;

P1->OUT    |=   BIT1;

P1->IES      |=   BIT1;           	//Set pin interrupt to trigger from high to low (starts high due to pull up resistor)

P1->IE        |=   BIT1;            	//Set interrupt on for P1.4

P1->SEL0 &=~ BIT4;          	// Setup the P1.4 on the Launchpad as Input, Pull Up Resistor

P1->SEL1 &=~ BIT4;

P1->DIR   &=~ BIT4;

P1->REN    |=   BIT4;

P1->OUT    |=   BIT4;

P1->IES      |=   BIT4;            	//Set pin interrupt to trigger from high to low (starts high due to pull up resistor)

P1->IE        |=   BIT4;            	//Set interrupt on for P1.1

P1->IFG = 0;                		//Clear all interrupt flags

}

SysTick Timer

| Offset | Acronym | Register name | Type | Reset | Section |
| --- | --- | --- | --- | --- | --- |
| 10h | STCSR | SysTick Control and Status Register | r/w | 00000004h | 2.4.4.1 |
| 14h | STRVR | SysTick Reload Value Register | r/w | Undefined | 2.4.4.2 |
| 18h | STCVR | SysTick Current Value Register | r/w |  | 2.4.4.3 |
| 1Ch | STCR | SysTick Calibration Value | r/w |  | 2.4.4.4 |

SysTick Registers

SysTick -> CTRL            (to access the Control Status Register STCSR)

SysTick -> LOAD            (to access the Reload Value Register STRVR)

SysTick -> VAL               (to access the Current Value Register STCVR

STCSR Register [SysTick -> CTRL]

SysTick Control and Status Register. Use the SysTick Control and Status Register to enable the SysTick features.

SysTick -> CTRL = 0x00000005;        	// enable SysTick, 3MHz, No Interrupts

SysTick -> CTRL |= BIT0 | BIT2;

STRVR Register [SysTick -> LOAD]

Specify the start value to load into the current value register when the counter reaches 0. It can be any value between 1 and 0x00FFFFFF. A start value of 0 is possible, but has no effect because the SysTick interrupt and COUNTFLAG are activated when counting from 1 to 0.

SysTick -> LOAD = 0x00FFFFFF;         	// maximum reload value

STCVR Register [SysTick -> VAL]

Use the SysTick Current Value Register to find the current value in the register.

Writing to it with any value clears the register to 0

SysTick -> VAL = 0;                  // any write to current clears it

Example

SysTick -> CTRL            (to access the Control Status Register STCSR)

SysTick -> LOAD            (to access the Reload Value Register STRVR)

SysTick -> VAL               (to access the Current Value Register STCVR

void SysTick_Init (void)

{        						  //initialization of SysTick timer

SysTick -> CTRL    = 0;                 		 // disable SysTick during step

SysTick -> LOAD  = 0x00FFFFFF;         	// max reload value

SysTick -> VAL      = 0;                   	// any write to current clears it

SysTick -> CTRL   = 0x00000005;        	// enable SysTick, 3MHz, No Interrupts

}

void SysTick_msdelay (uint16_t msdelay)		       // SysTick delay function

{

SysTick -> LOAD = ((delay * 3000) - 1);                    //delay for 1 ms* delay value

SysTick -> VAL     = 0;                                                 // any write to CVR clears it

while ( (SysTick - > CTRL  &  0x00010000) == 0);  // wait for flag to be SET

}

void SysTick_usdelay (uint16_t usdelay)		       // SysTick delay function

{

SysTick -> LOAD = ((delay * 3) - 1);                          //delay for 1 us * delay value

SysTick -> VAL     = 0;                                                 // any write to CVR clears it

while ( (SysTick - > CTRL  &  0x00010000) == 0);  // wait for flag to be SET

}

For 3 MHz frequency

Period T = 1/f

T = 1 / 3000000

T = 0.333 us = 333.333 ns   				so each cycle = 1 period = 0.333 us

1 ms = 1x103 / 0.333 us

1 ms = 3000 cycles

With Interrupts

Example:

Toggle pin 4 PORT 2 every 1 second Using SysTick and interrupts

We will use a reload value of 500 * 3000 (1 ms) to measure 0.5 second

volatile uint32_t timeout ;

int main(void)

{

WDT_A->CTL = WDT_A_CTL_PW | WDT_A_CTL_HOLD;	// Halting the Watchdog

P2->SEL0 &=~ BIT4 ;			// Set pin 4  GPIO

P2->SEL1 &=~ BIT4 ;			// Set pin 4  GPIO

P2->DIR      |=   BIT4 ;			// Set pin 4  OUTPUT

SysTick_Init_interrupt ( );        		// initialize SysTick with interrupt

__enable_irq ( ); 				// enable global interrupts

while (1)

{

if ( timeout )

{

P2->OUT ^= BIT4;		// Toggle pin 4 every second

timeout = 0;

}

}

}

void SysTick_Init_interrupt (void)

{        						  //initialization of SysTick timer

SysTick -> CTRL    = 0;                 		 // disable SysTick during step

SysTick -> LOAD   = 1500000;		// Value for 0.5s interrupts interrupt

SysTick -> VAL      = 0;                   		// any write to current clears it

SysTick -> CTRL   = 0x00000007;        	// enable SysTick, 3MHz, with Interrupt

}

void SysTick_Handler (void)		//SysTick handler interrupt

{

timeout = 1;              			// set flag =1, reset in main

}

void SysTick_Handler (void)		//SysTick handler interrupt

{

count++;          //increment count… set as global variable before main

}

Timer A Registers

#define TIMER_A_CTL_TASSEL_0		// TAxCLK	#define TIMER_A_CTL_ID_0 // divide by 1

#define TIMER_A_CTL_TASSEL_1        		// ACLK 	#define TIMER_A_CTL_ID_1 // divide by 2

#define TIMER_A_CTL_TASSEL_2        		// SMCLK 	#define TIMER_A_CTL_ID_2 // divide by 4

#define TIMER_A_CTL_TASSEL_3        		// INCLK 	#define TIMER_A_CTL_ID_3 // divide by 8

#define TIMER_A_CTL_SSEL__TACLK    	// TAxCLK

#define TIMER_A_CTL_SSEL__ACLK      	// ACLK

#define TIMER_A_CTL_SSEL__SMCLK     	// SMCLK

#define TIMER_A_CTL_SSEL__INCLK     	// INCLK

#define TIMER_A_CTL_MC_0            		// Stop mode: Timer is halted

#define TIMER_A_CTL_MC_1            		// Up mode: Timer counts up to TAxCCR0

#define TIMER_A_CTL_MC_2            		// Continuous mode: Timer counts up to 0FFFFh

#define TIMER_A_CTL_MC_3            		// Up/down mode: Timer counts up to TAxCCR0 then

#define TIMER_A_CTL_MC__STOP      		// Stop mode: Timer is halted

#define TIMER_A_CTL_MC__UP          		// Up mode: Timer counts up to TAxCCR0

#define TIMER_A_CTL_MC__CONTINUOUS 	// Continuous mode: Timer counts up to 0FFFFh

#define TIMER_A_CTL_MC__UPDOWN      	// Up/down mode: Timer counts up

TIMER_A0->CTL = TIMER_A_CTL_ TASSEL_2 |   	 // SMCLK

TIMER_A_CTL_MC_1 |       	 // Up Mode ... Count up

TIMER_A_CTL_CLR;         		 // clear TA0R Register

TIMER_A0->CTL |= TASSEL_2 | MC_1 | TACLR; 	// SMCLK, Up Mode (Counts to TA0CCR0)

TA0CTL 	  |= TASSEL_2 | MC_1 | TACLR;		// SMCLK, Up Mode (Counts to TA0CCR0)

TIMER_A0->CTL |= 0b0000 0010 0001 0100;		// SMCLK, Up Mode (Counts to TA0CCR0)

TIMER_A0->CTL |= 0x0214;				// SMCLK, Up Mode (Counts to TA0CCR0

Timer/counter register

16-Bit Timer Counter

The 16-bit timer/counter register, TAxR, increments or decrements (depending on mode of operation) with each rising edge of the clock signal. TAxR can be read or written with software. Additionally, the timer can generate an interrupt when it overflows.

TAxR may be cleared by setting the TACLR bit. Setting TACLR also clears the clock divider and count direction for up/down mode.

Timer_A Capture/Compare 0 Register

Changing Period Register TAxCCR0

When changing TAxCCR0 while the timer is running, if the new period is greater than or equal to the old period or greater than the current count value, the timer counts up to the new period. If the new period is less than the current count value, the timer rolls to zero. However, one additional count may occur before the counter rolls to zero.

Example of a period 1000;

TIMER_A0-> CCR[0]  = 1000;         // PWM Period

Or

TA0CCR0 |= 1000;			 // PWM Period

Timer_Ax Capture/Compare Control 0 Register to Timer_Ax Capture/Compare Control 6

CCTL

#define TIMER_A_CCTLN_OUTMOD_0 	// OUT bit value 		#define TIMER_A_CCTLN_CAP_OFS   // Compare mode

#define TIMER_A_CCTLN_OUTMOD_1 	// Set 			#define TIMER_A_CCTLN_CAP    	// Capture mode

#define TIMER_A_CCTLN_OUTMOD_2 	// Toggle/reset

#define TIMER_A_CCTLN_OUTMOD_3 	// Set/reset

#define TIMER_A_CCTLN_OUTMOD_4 	// Toggle

#define TIMER_A_CCTLN_OUTMOD_5 	// Reset

#define TIMER_A_CCTLN_OUTMOD_6 	// Toggle/set

#define TIMER_A_CCTLN_OUTMOD_7 	// Reset/set

Example Reset/Set mode, capture mode

TIMER_A0-> CCTL[1] = TIMER_A_CCTLN_OUTMOD_7; 	// Reset/set

TA0-> CCTL1 |= OUTMOD_7;				  	// TA0CCR1 output mode = reset/set

TIMER_A0->CCTL[1] = 0b0000 0000 1110 0000; 			// Reset/set capture compare mode

TIMER_A0->CCTL[1] = 0x00E0;					// Reset/set

Timer_Ax Interrupt Vector Register

Timer_Ax Expansion 0 Register

MSP432 TimerA Example

PWM example using both TimerA_0 and TimerA_1 */

#include< msp.h>

void port_init (void);

void TimerA_init (void);

int main (void)

{

WDT_A->CTL = WDT_A_CTL_PW | WDT_A_CTL_HOLD;     // stop watchdog timer

port_init ( );

TimerA_init ( );

while (1)

{

;       //Timer A is connected to 2.4 and 7.7, if you connect an LED to these pins they will

// have different intensities

}

}

void port_init (void)		// GPIO Set-Up

{

P2->SEL0     |=    BIT4;		// P2.4 selected TA0

P2->SEL1   &= ~BIT4;		// P2.4 selected TA0

P2->DIR       |=    BIT4;		// P2.4 set as ou

P7->SEL0     |=    BIT7;		// P7.7 selected TA1

P7->SEL1   &=~ BIT7;		// P7.7 selected TA1

P7->DIR       |=    BIT7;		// P7.7 set as output

}

void TimerA_init (void)		//Timer0_A Set-Up

{

TIMER_A0->CCR[0]    |= 200 - 1;				// PWM Period

TIMER_A0->CCTL[1] |= OUTMOD_7;			// TA0CCR1 output mode = reset/set

TIMER_A0->CCR[1]    |= 100;				// TA0CCR1 PWM 50% duty cycle

TIMER_A0->CTL         |= TASSEL_2 | MC_1 | TACLR;		// SMCLK, Up mode, Clear

TIMER_A1->CCR[0]     |= 1000 - 1;				// PWM Period

TIMER_A1->CCTL[1]   |= OUTMOD_7;			// TA1CCR1 output mode = reset/set

TIMER_A1->CCR[1]     |= 500;				// TA1CCR1 PWM duty cycle

TIMER_A1->CTL          |= TASSEL_2 | MC_1 | TACLR;		// SMCLK, Up Mode, Clear

}

OR you can write in Binary form

TIMER_A0->CTL         |= 0b 0000 0010 0001 0100;		// SMCLK, Up mode, Clear

TIMER_A1->CTL         |= TASSEL_2 | MC_1 | TACLR;	// SMCLK, Up Mode, Clear

/**************************************************************

*

*              Nabeeh Kandalaft

*    Connect an LED to pin P2.4 and control the intensity of the LED by

*                                                 using TIMERA0 PWM

**************************************************************/

#include "driverlib.h"

void PWM_init(void);

volatile int period        = 64000 ;     //variable to set the 20 ms period ....3200 ==> 1 ms on 3MHz clock

volatile int duty_cycle =3200;	     // variable to set the ON Time for 5% Duty Cycle

int main(void)

{

WDT_A->CTL = WDT_A_CTL_PW | WDT_A_CTL_HOLD;     	// stop watchdog timer

PWM_init( );

while (1)

{

TIMER_A0->CCR[1] = duty_cycle;        //Set breakpoint here

}

}

void PWM_init( void)

{

P2->SEL0     |=    BIT4;    	 // P2.4 selected TA0

P2->SEL1   &= ~BIT4;     	//  P2.4 selected TA0

P2->DIR       |=    BIT4;     	//  P2.4 set as output

TIMER_A0->CCR[0]   =  period;                        	 		// PWM period

TIMER_A0->CCR[1]   =  duty_cycle;                    			// Initial duty cycle

TIMER_A0->CCTL[1] = TIMER_A_CCTLN_OUTMOD_7;         // Reset/set

TIMER_A0->CTL        =  TIMER_A_CTL_TASSEL_2  |         	// SMCLK

TIMER_A_CTL_MC_1  |            		// Up Mode ... Count up

TIMER_A_CTL_CLR;                		// clear TA0R Register

}

The continuous mode can be used to generate independent time intervals and output frequencies. Each time an interval is completed, an interrupt is generated. The next time interval is added to the TAxCCRn register in the interrupt service routine

This is controlled in the CCTL register

For the over flow 0XFFFF interrupt when it reaches the maximum count

It is controlled in CTL register

Toggle P1.0 using software and the Timer0_A overflow ISR.

In this example an ISR triggers when TA overflows. (Hint…this means using the CTL interrupt flag)

Inside the ISR P1.0 is toggled. Toggle rate is exactly 0.5Hz which is 2 seconds.

Hint…this means using the ACLK clock 32768 Hz

T = 1/ 32768

T = 30.51us …… over flow occur at 65535 * 30.51us …… 2 seconds which is 0.5 Hz

#include "msp.h"

int main(void)

{      WDT_A->CTL = WDT_A_CTL_PW |  WDT_A_CTL_HOLD;	// stop watchdog timer

void Port_init ( );  					 	// LED Initialization on PORT1 pin 0

TIMER_A0_init ( );							// TIMER_A0 initialization

NVIC->ISER[0] = 1 << ((TA0_0_IRQn) & 31);			// Enable interrupt in NVIC vector

__enable_irq ( );							// Enable global interrupt

while (1)

{

; 	            // Hardware is doing all the work, so nothing more to do in this while loop

}

}

void Port_init (void)

{

P1->SEL0 &=~ BIT0;

P1->SEL1 &=~ BIT0;

P1->DIR     |=    BIT0;

P1->OUT &=~  BIT1;

}

void TIMER_A0_init (void)

{

TIMER_A0->CTL = TIMER_A_CTL_SSEL__ACLK | 		// ACLK 32768 Hz

TIMER_A_CTL_MC__CONTINUOUS |	// Continuous Mode   MC_2

TIMER_A_CTL_CLR |            		 // Clear TAR

TIMER_A_CTL_IE;               		// Enable timer overflow interrupt

}

void TA0_0_IRQHandler(void)

{

P1->OUT ^= BIT0;							// Toggle P1.0 LED

TIMER_A0->CTL[0] &= ~ TIMER_A_CTL_TAIFG;		// Clear the flag in CCTL[0]

}

Toggle P1.0 using software and TA_0 Interrupt Service Routine.

Timer0_A is configured for continuous mode, thus the timer overflows when TAR counts to CCR0.

In this example, CCR0 is loaded with 50000. MCLK = SMCLK = 3MHz

#include "msp.h"

int main(void)

{

WDT_A->CTL = WDT_A_CTL_PW |  WDT_A_CTL_HOLD;	// stop watchdog timer

void Port_init ( )  					 // LED Initialization on PORT1 pin 0

TIMER_A0_init ( );					// TIMER_A0 initialization

NVIC->ISER[0] = 1 << ((TA0_0_IRQn) & 31);	// Enable interrupt in NVIC vector

__enable_irq ( );					// Enable global interrupt

while (1)

{

; 	            // Hardware is doing all the work, so nothing more to do in this while loop

}

}

void TIMER_A0_init (void)

{

TIMER_A0->CCR[0]   = 50000;

TIMER_A0->CTL = TIMER_A_CTL_SSEL__SMCLK | 		// SMCLK

TIMER_A_CTL_MC__CONTINUOUS;	// Continuous Mode   MC_2

TIMER_A0->CCTL[0] = TIMER_A_CCTLN_CCIE; 		// TACCR0 interrupt enabled

}

void Port_init (void)

{

P1->SEL0 &=~ BIT0;

P1->SEL1 &=~ BIT0;

P1->DIR     |=    BIT0;

P1->OUT &=~  BIT1;

}

void TA0_0_IRQHandler(void)

{

P1->OUT ^= BIT0;							// Toggle P1.0 LED

TIMER_A0->CCTL[0] &= ~ TIMER_A_CCTLN_CCIFG;	// Clear the flag in CCTL[0]

}

Example:  Scott Zuidema

Sets up A0.1 as timer output to P2.4.  Sets up A0.4 as timer input (capture on P2.7) on both rising/falling edges

If P2.4 and P2.7 are connected the same TIMER_A0, LED1 will flash twice as fast as LED2. LED2 turns on/off with each Timer compare match.

(When the Timer compare matches) and P2.4 on (which happens on Timer A0 overflow)

#include "msp.h"

void main(void)   {

WDT_A->CTL = WDT_A_CTL_PW | WDT_A_CTL_HOLD;		// stop watchdog timer

Port_init ( );

Timer_A0_init ( );

NVIC->ISER[0] = 1 << ((TA0_0_IRQn) & 31);	// Enable interrupt in NVIC vector

__enable_irq ( );					// Enable global interrupt

while (1)

;		// Hardware is doing all the work, so nothing more to do in this while loop.

}

// Clock is 3 MHz, the setup makes timer A0 have a clock of 3 MHz / 64 = 46.875 kHz.

// Timer A0 counts to 65535 which means it overflows once every 1.39 seconds. Each step is 0.00002133 s or 21.33 us.

void Timer_A0_init (void)

{

TIMER_A0->CTL       = 0b0000001011010100;     	 // SMCLK, Divide by 8, Count Up, Clear to start

TIMER_A0->CCR[0]   = 65535;      			 // A0 count to 2^16 - 1.  Set to 46875 for a 1 second flash rate.

TIMER_A0->CCR[1]   = 32767;                       		 // Duty cycle to 50% to start (1/2 of CCR0).

TIMER_A0->CCTL[1] = 0b0000000011110100;        	 // CCR1 reset/set mode 7, with interrupt.

TIMER_A0->CCTL[4] = 0b1100100100010100;              	 // Capture on rising/falling edge. *Compared to CCIA (ignore)

// synchronized, interrupt enabled

//TIMER_A0-> CCTL[4] = 0b0100100100010100;      	 // Alternative.  This set it up for only capture on rising edge.

TIMER_A0->EX0         = 0b0000000000000111;           	 // divide the clock by 1+EX0, in this case, a divider of 8    						.

TIMER_A0->CCTL[1] &=~ BIT0;                     		// Clear Timer A0.1 interrupt flag

TIMER_A0->CCTL[1] &=~ BIT1;                     		// Clear Timer A0.1 overflow flag

TIMER_A0->CCTL[4] &=~ BIT0;                     		// Clear Timer A0.4 interrupt flag

TIMER_A0->CCTL[4] &=~ BIT1;                     		// Clear Timer A0.4 overflow flag

}

// Breaking down the setup of CCTL [4] by bit

// Bits 15 - 14 = 11        capture interrupt happens on both rising and falling edges.

// Bits 13 - 12 = 00        CCIxA is a reference.  Data sheet states that P2.7 is connected to CCIA, so that is selected

// Bit 11          = 1          Synchronize. But if things don't occur on clock edges, bad things may happen in future programs.  // Bit 10          = 0          A little fuzzy on this, but apparently if things are synchronized, I can read it here later

// Bit 9            = 0          Reserved.  Either TI uses it when testing, there is a feature on other processors but not

// Bit 8            = 1          the magic bit to enable capture (instead of compare)

// Bit 7 - 5      = 000      Use these when doing a compare, not for capture

// Bit 4           = 1          Enabled the interrupt!

// Bit 3           = 0           Read only.  You can read the current state of P2.7

// Bit 2           = 1          Also for compare.  Doesn't do much here.  Does the same thing for 0 or 1.

// Bit 1           = 0          Clear the overflow flag

// Bit 0           = 0          Clear the interrupt flag (don't want it to be on until we actually get a real interrupt).

void TA0_N_IRQHandler ( ) {             		// Timer A0 interrupt happened

if (TIMER_A0->CCTL[4] & BIT0)  { 		// T0.4 was the cause.  Successful capture means blink LED1.

TIMER_A0->CCTL[4] &=~ BIT0;  		// Clear Timer A0.4 interrupt flag

TIMER_A0->CCTL[4] &=~ BIT1;   	// Clear Timer A0.4 overflow flag

P1->OUT ^= BIT0;                              	// Blinking LED1

}

if (TIMER_A0->CCTL[1] & BIT0)  {  		// T0.1 was the cause.  Successful compare means blink LED2.

TIMER_A0->CCTL[1] &=~ BIT0;    	// Clear Timer A0.1 interrupt flag

TIMER_A0->CCTL[1] &=~ BIT1;  		// Clear Timer A0.1 overflow flag

P2->OUT ^= BIT0;                             	// Blinking LED2

}

}

void Port_init (void)

{

P1->SEL0 &=~ BIT0;		// Setup GPIO

P1->SEL1 &=~ BIT0;

P1->DIR      |=   BIT0;     	// Setup P1.0 as an OUTPUT, initialize to 0

P1->OUT &=~  BIT0;

P2->SEL0 &=~ BIT0;		// Setup GPIO

P2->SEL1 &=~ BIT0;

P2->DIR      |=   BIT0;   	// Setup P2.0 as an OUTPUT, initialize to 0

P2->OUT &=~  BIT0;

P2->SEL0   |=   BIT4;

P2->SEL1 &=~ BIT4;

P2->DIR      |=   BIT4;       	// Setup P2.4 as an TimerA0 controlled OUTPUT, SEL = 01

P2->SEL0   |=   BIT7;

P2->SEL1 &=~ BIT7;

P2->DIR   &=~ BIT7;     	// Setup P2.7 an INPUT connected to Timer A0.4 for Capture!  SEL = 01

}

Analog to digital conversion

ADC

The ADC14 module supports fast 14-bit analog-to-digital conversions. The module implements a 14-bit in SAR core, sample select control, and up to 32 independent conversion-and-control buffers.

- The ADC14 core is configured by two control registers, ADC14CTL0 and ADC14CTL1.

- The core is reset when ADC14ON = 0.

- When ADC14ON = 1, reset is removed and the core is ready to power up when a valid conversion is triggered.

- The ADC14 can be turned off when not in use to save power.

- If during a conversion the ADC14ON bit is set to 0, the conversion is abruptly exited, ADC is powered down.

- With few exceptions, the ADC14 control bits can be modified only when ADC14ENC = 0.

- ADC14ENC must be set to 1 before any conversion can take place.

- The conversion results are always stored in binary unsigned format.

- For differential inputs, this means that the result has an offset of 8192 added to it to make the number positive. The data format bit ADC14DF in ADC14CTL1 allows the user to read the conversion results as binary unsigned or signed binary (2s complement).

- ADC14CLK is used for the conversion clock and to generate sampling period in pulse mode.

- The ADC14 source clock is selected using the ADC14SSELx bit.

- The input clock can be divided by 1, 4, 32, or 64 using the ADC14PDIV bits

- And then subsequently divided by 1 to 8 using the ADC14DIV bits.

- Possible ADC14CLK sources are MODCLK, SYSCLK, ACLK, MCLK, SMCLK, HSMCLK

- The analog-to-digital conversion requires 9, 11, 14 and 16 ADC14CLK cycles for 8-bit, 10-bit, 12-bit, and 14-bit resolution modes respectively

- There are 32 ADC14MEMx conversion memory registers to store conversion results.

- Each ADC14MEMx is configured with an associated ADC14MCTLx control register.

- The ADC14VRSEL bits define the voltage reference and the ADC14INCHx and ADC14DIF bits select the input channels.

When conversion results are written to a selected ADC14MEMx, the corresponding flag in the ADC14IFGRx register is set.

ADC14CTL0 Register

ADC14 Control 0 Register

Figure 20-12. ADC14CTL0 Register

ADC14CTL1 Register

ADC14 Control 1 Register

Figure 20-13. ADC14CTL1 Register

ADC14MCTL0 to ADC14MCTL31 Register

ADC14 Conversion Memory Control x Register (x = 0 to 31)

Figure 20-18. ADC14MCTL0 to ADC14MCTL31 Register

ADC14MEM0 to ADC14MEM31 Register

ADC14 Conversion Memory x Register (x = 0 to 31)

Figure 20-19. ADC14MEM0 to ADC14MEM31 Register

ADC14IER0 Register

ADC14 Interrupt Enable 0 Register

Figure 20-20. ADC14IER0 Register

ADC14IFGR0 Register (offset = 144h) [reset = 00000000h]

ADC14 Interrupt Flag 0 Register

Figure 20-22. ADC14IFGR0 Register

The ADC14IFGx bits are set when their corresponding ADC14MEMx memory register

is loaded with a conversion result

The rest of the flag bits can be found in the technical reference manual

ADC14CLRIFGR0 Register

ADC14 Clear Interrupt Flag 0 Register

Figure 20-24. ADC14CLRIFGR0 Register

ADC14IV Register

ADC14 Interrupt Vector Register

Figure 20-26. ADC14IV Register

ADC Examples

Connect a photo conductive cell (Photo Diode) in a simple analog circuit designed to create a voltage that increases with increased light level. This voltage will be displayed on the console

Use A0 ADC Pin5.5, 14 bit ADC, SMCLK for clock source, and 16 sample clocks

#include "msp.h"

void delaysetup(void);

void delayms(uint16_t delay);

void ADC14_init (void);

main(void)

{

static volatile uint16_t result;

float nADC;

delaysetup ( );

ADC14int ( );

WDT_A->CTL = WDT_A_CTL_PW | WDT_A_CTL_HOLD; 	// stop watchdog timer

while(1)

{

ADC14->CTL0 |= 1; 				//start conversation

while(!ADC14->IFGR0); 			//wait for conversation to complete

result = ADC14->MEM[0]; 			// get the value from the ADC

nADC = (result * 3.3) / 16384;

printf("Value is:\n\t%d\n\t%f\n",result, nADC);

delayms(2000);

}

}

void delaysetup (void) 					// sets up the countdown timer

{

SysTick-> CTRL = 0;

SysTick-> LOAD = 0x00FFFFFF;

SysTick-> VAL    = 0;

SysTick-> CTRL = 0x00000005;

}

void delayms(uint16_t delay) 				// timer function set to run a 1 ms increments

{

SysTick->LOAD = ((delay * 3000)-1);

SysTick->VAL = 0;

while((SysTick-> CTRL & 0x00010000) == 0);

}

void ADC14_init (void)

{

P5SEL0 |= 0X20; 					// configure pin 5.5 for A0 input

P5SEL1 |= 0X20;

ADC14->CTL0 &=~ 0x00000002; 			// disable ADC14ENC during configuration

ADC14->CTL0   |=  0x04200210; 			// S/H pulse mode, SMCLK, 16 sample clocks

ADC14->CTL1    =   0x00000030; 			// 14 bit resolution

ADC14->CTL1   |=   0x00000000; 			// Selecting ADC14CSTARTADDx mem0 REGISTER

ADC14->MCTL[0] = 0x00000000;			// ADC14INCHx = 0 for mem[0]

// ADC14->MCTL[0] = ADC14_MCTLN_INCH_0;

ADC14->CTL0 |= 0x00000002;			// enable ADC14ENC, starts the ADC after configuration

}

Example

Measure ambient light intensity with the photocell and microcontroller ADC and use that information to control the light intensity of the LED such that when the ambient light decreases (it gets darker) the intensity of the LED also decreases (it gets dimmer). Use the same parameters from last example. Connect the LED to pin P2.7 for TIMER_A0

#include "msp.h"

void delaysetup (void);

void delayms (uint16_t delay);

void ADC14_int (void);

void port_init (void);

void PWMsetup (void);

main(void)

{

static volatile uint16_t result;

float nADC, bright;

delaysetup ( );

ADC14int  ( );

pintsetup   ( );

PWMsetup();

WDT_A->CTL = WDT_A_CTL_PW | WDT_A_CTL_HOLD;	 // stop watchdog timer

while(1)

{

ADC14->CTL0 |= ADC14_CTL0_SC;		//start conversation or write ADC14->CTL0 |= 1;

while ( ( !ADC14->IFGR0 & BIT0 )  );		//wait for conversion to complete

result  = ADC14->MEM[0];			// get the value from the ADC

nADC = (result * 3.3) / 16384;

delayms ( 500 );

printf ("Value is:\n\t%d\n\t%f\n", result, nADC );

bright = nADC / 3 * 50000; 	// 3 was the highest voltage from the cell not 3.3.

//Always replace by the highest value

TIMER_A0->CCR[4] = bright;

}

}

void delaysetup (void) {					 //sets up the countdown timer

SysTick-> CTRL = 0;

SysTick-> LOAD = 0x00FFFFFF;

SysTick-> VAL    = 0;

SysTick-> CTRL = 0x00000005;

}

void delayms (uint16_t delay) {				 //timer function set to run a 1 ms increments

SysTick-> LOAD = ((delay * 3000)-1);

SysTick-> VAL    = 0;

while((SysTick-> CTRL & 0x00010000) == 0);

}

void ADC14_init (void) {

P5SEL0               |= 0X20; 				// configure pin 5.5 for A0 input

P5SEL1               |= 0X20;

ADC14->CTL0 &=~ 0x00000002; 			// disable ADC14ENC during configuration

ADC14->CTL0   |=   0x04200210; 			// S/H pulse mode, SMCLK, 16 sample clocks

ADC14->CTL1    =   0x00000030; 			// 14 bit resolution

ADC14->CTL1   |=   0x00000000; 			// Selecting ADC14CSTARTADDx mem0 REGISTER

ADC14->MCTL[0] = 0; 				// ADC14INCHx = 0 for mem[0]

ADC14->CTL0 |= 0x00000002;				// enable ADC14ENC, starts the ADC after configuration

}

void Port_init (void) {

P2->SEL0   |=   BIT7;					// TIMER_A_4 will be connected to led on pin p2.7

P2->SEL1 &= ~BIT7;

P2->DIR     |=   BIT7;

}

void PWMsetup (void) {

TIMER_A0->CCR[0]   = 50000-1; 			//period   f = 60Hz Ambient light

TIMER_A0->CCR[4]   = 500; 				//duty cycle 100% = 50000 500 = 1%

TIMER_A0->CCTL[4] = 0xE0;			//CCR4 reset/set mode TIMER_A_CCTLN_OUTMOD_7

TIMER_A0->CTL        = 0x0214;			//use SMCLK, count up, clear TAOR register

}

Example:

- A single sample is made on A1 with reference to AVcc.  (A1 Is P5.4      LED is on P1.0)

- Software sets ADC14_CTL0_SC to start sample and conversion

- ADC14_CTL0_SC automatically cleared at EOC (End of conversion).

- ADC14 internal oscillator times sample (16x) and conversion.

- MSP432 waits in LPM0 (Low Power Mode) to save power until ADC14 conversion complete

- ADC14_ISR will force exit from LPM0 in Main loop

- If A1 > 0.5 * AVcc, P1.0 set, else reset.

#include "msp.h“

int main(void)

{

WDT_A->CTL = WDT_A_CTL_PW  |  WDT_A_CTL_HOLD;            // Stop WDT

Port _init ( );

ADC_init ( );

NVIC->ISER[0] = 1 << ((ADC14_IRQn) & 31);  		 // Enable ADC interrupt in NVIC module

__enable_irq ( );                             				// Enable global interrupt

while (1)

{	// you put a small delay here

ADC14->CTL0 |= ADC14_CTL0_SC; 			// Start conversion

__sleep ( );							// Go to low power mode

}

}

void Port_init (void)

{

P1->SEL0 &=~ BIT0;		// Set GPIO

P1->SEL1 &=~ BIT0;

P1->DIR     |=    BIT0;		// Set P1.0/LED to output

P1->OUT &=~  BIT1;		// Clear LED to start

P5->SEL0  |=    BIT4;		// Set  ADC GPIO

P5->SEL1  |=    BIT4;		// Configure P5.4 for ADC

}

void ADC_init (void)

{

ADC14 -> CTL0  & =~  ADC_CTL0_ENC;			// disable ADC converter during initialization 	                                   	                    	// ADC14 ON                 S&H=16,                          Use sampling timer

ADC14->CTL0        = ADC14_CTL0_ON  | ADC14_CTL0_SHT0_2 | ADC14_CTL0_SHP ;

ADC14->CTL1        = ADC14_CTL1_RES_3;         		// 14-bit conversion results

ADC14->MCTL[1] |= ADC14_MCTLN_INCH_1;  	 	// A1 ADC input select; Vref = AVCC

ADC14->IER0        |= ADC14_IER0_IE1;          			// Enable ADC conv complete interrupt

ADC14 -> CTL0     |= ADC_CTL0_ENC;			// Enable ADC converter

}

void ADC14_IRQHandler (void)

{

if (ADC14->MEM[1] >=  0x2000 )             			// ADC12MEM1 = A1 > 0.5AVcc?

P1->OUT |= BIT0;					// P1.0 LED ON

else

P1->OUT ^= BIT0;					// P1.0 LED OFF

}

Example

Changing the period on a Piezo Buzzer using ADC and TIMERA_0

Done by Karl Brokara

#include "msp.h"

#include <stdio.h>

#define ADC_PORT	P5			//ADC Input Port

#define ADC_PIN	BIT5			//ADC Input Pin

#define ADC_INST	0			//Which instance of ADC

#define SPEAKER_PORT	P2		Speaker PWM Port

#define	SPEAKER_PIN		BIT4		//Speaker PWM Pin

#define	SPEAKER_INST	1		//TIMER A0 Instance for speaker

#define SYSTICK_TICKS	30000		//Which instance of

void ADC14init(void);

void SysTickInit(void);

void TimerA0config(void);

void SpeakerConfig(void);

uint16_t	adc_value = 1023;			//ADC Measurement set to adc

void main(void)

{

WDT_A->CTL = WDT_A_CTL_PW | WDT_A_CTL_HOLD;	// stop watchdog timer

__disable_irq();

SysTickInit();		//Init SysTick

SpeakerConfig();	//Set PWM Pins to drive Speaker

TimerA0config();	//Configure Timer A0 to make tone

ADC14init();		//Start ADC to read from potentiometer

NVIC->ISER[0]	|=1<<ADC14_IRQn;	//Turn on ADC Interrupts in NVIC

__enable_irq();

while(1);//Loop forever

}

void ADC14init(void)

{

/* Function will set up the ADC14 to run in single measurement mode and to interrupt

* upon conversion.

* Clock Source: SMCLK

* Clock DIV:	32

* Resolution: 10 bits

*/

ADC_PORT->SEL0 	|= 	 ADC_PIN;					//Select ADC Operation

ADC_PORT->SEL1 	|= 	 ADC_PIN;

ADC14->CTL0	&=~ADC14_CTL0_ENC;			//Turn of Enable Conversion

ADC14->CTL0	|=ADC14_CTL0_PDIV__32 |  			//Set PDIV to 32

ADC14_CTL0_SHP |				//Sample and Hold off of Sampling timer

ADC14_CTL0_SSEL__SMCLK |   		//SMCLK

ADC14_CTL0_SHT1__32	|		//Hold memory register for 32 clock cycles

ADC14_CTL0_ON;				//Turn on the ADC

ADC14->CTL1|=  ADC14_CTL1_RES__10BIT;			//Set to 10-bit resolution

ADC14->MCTL[ADC_INST]= 	 0;				//Default configuration for ADC Channel

ADC14->IER0	|=	 ADC14_IER0_IE0;			//Interrupt on

ADC14->CTL0	|=	 ADC14_CTL0_ENC;			//Enable Conversion

}

void SysTickInit(void)

{

SysTick->LOAD	=	SYSTICK_TICKS;	//Set Interval

SysTick->VAL	= 	0;			//Reset Value

SysTick->CTRL	= 	7;			//Set CLK, Set IE, Set run

}

void TimerA0config(void)

{

TIMER_A0->CCR[0] = 0xFFFF;      						//Set the period of Timer0 PWM

TIMER_A0->CCR[SPEAKER_INST] = 0;					//Set the period of Timer0 PWM

TIMER_A0->CCTL[SPEAKER_INST] = TIMER_A_CCTLN_OUTMOD_7;	//Set to Reset/set Compare Mode

TIMER_A0->CTL = TIMER_A_CTL_TASSEL_2  |             			 //SMCLK and Input divider 1

TIMER_A_CTL_MC__UP     |

TIMER_A_CTL_CLR;    					 //Up mode and clear the timer

}

void SpeakerConfig(void)

{

SPEAKER_PORT->SEL0 |= 	SPEAKER_PIN;	//Config to TIMER_A0 PWM Control

SPEAKER_PORT->SEL1 &= ~SPEAKER_PIN;

SPEAKER_PORT->DIR |= 	SPEAKER_PIN;	//Set to output

}

void SysTick_Handler(void)

{

ADC14->CTL0	|=ADC14_CTL0_SC;			//Start ADC Conversion

}

void ADC14_IRQHandler(void)

{

if(ADC14->IV == (0x0000000C+2*ADC_INST))			//If interrupted by ADC_INST

{

TIMER_A0->CCR[0] = 100*(600 + 15*(ADC14->MEM[ADC_INST]>>4))/100;		//Set Period of speaker

TIMER_A0->CCR[SPEAKER_INST]	= TIMER_A0->CCR[0]/2;		//Set duty cycle to 50%

}

ADC14->IV= 0;		//Clear ADC interrupt flags

}

Enhanced Universal Serial Communication Interface

(eUSCI) – UART Mode

Initializing or reconfiguring the eUSCI_A module

The recommended eUSCI_A initialization/reconfiguration process is:

1. Set UCSWRST.

2. Initialize all eUSCI_A registers with UCSWRST = 1 (including UCAxCTL1).

3. Configure ports.

4. Clear UCSWRST with software. Clearing UCSWRST releases the eUSCI_A for operation.

5. Enable interrupts (optional) with UCRXIE or UCTXIE.

Setting a Baud Rate

For a given BRCLK clock source (Baud rate Clock Source), the baud rate used determines the required division factor N:

N = fBRCLK / baud rate

The division factor N is often a non-integer value, thus, at least one divider and one modulator stage is

Used to meet the factor as closely as possible.

If N is equal or greater than 16, it is recommended to use the oversampling baud-rate generation mode by setting UCOS16.

Baud-rate settings quick set up

To calculate the correct settings for the baud-rate generation, perform these steps:

- Calculate N = fBRCLK / baud rate [if N > 16 	[continue with step 3, otherwise with step 2]

- OS16 = 0		 UCBRx  = INT(N) 		[continue with step 4]

3.    OS16 = 1 		UCBRx   = INT (N/16),

UCBRFx = INT ([(N/16) – INT (N/16)] × 16)

4.    UCBRSx can be found by looking up the fractional part of

N = N - INT (N) 		 Table 22-4

5.    If OS16 = 0 was chosen, a detailed error calculation is recommended to be performed

Table 22-4. 	UCBRSx Settings for Fractional Portion of N = fBRCLK/Baud Rate

eUSCI_A has only one interrupt vector that is shared for transmission and for reception.

UCTXIFG interrupt flag is set by the transmitter to indicate that UCAxTXBUF is ready to accept another character.

UCTXIFG is automatically reset if a character is written to UCAxTXBUF.

UCTXIFG is set after a Hard Reset or when UCSWRST = 1.

An interrupt request is generated if UCTXIE is set.

UCTXIE is reset after a Hard Reset or when UCSWRST = 1.

UCAxCTLW0 Register … eUSCI_Ax Control Word Register 0

Figure 22-12. UCAxCTLW0 Register

v

Example

EUSCI_A0->CTLW0     = 0x0001; 			 // hold the USCI module in reset mode

EUSCI_A0->CTLW0     = 0x00C1;

EUSCI_A0->CTLW0  &= ~0x0001; 			 // enable the USCI module

UCAxBRW Register …. eUSCI_Ax Baud Rate Control Word Register

Figure 22-14. UCAxBRW Register

EUSCI_A0->BRW = 26;          	// UCBR = baud rate = int(N) 	= clock/baud rate

=clock/baud rate = 3MHz / 115200

= 26.0417

UCAxMCTLW Register …. eUSCI_Ax Modulation Control Word Register

Figure 22-15. UCAxMCTLW Register

EUSCI_A0->MCTLW &= ~0xFFF1; 		 // clear first and second modulation stage bit fields

Example with UCOS16 enabled

Baud rate 9600	Frequency 12000000

N = 12000000/9600 = >1250

OS16        = 1

UCBRx    = INT(N/16) = INT (12000000/(16*9600))

= 78

UCBRFx = INT(   [ ( N / 16)     – INT(N/16)        ] x 16   )

= INT(   [ (1250 / 16) – INT(71250/16) ] x 16   )

= INT(   [        78.125 - 78                      ] x 16  )

= INT(   [                0 .125                       ] x 16  )

= 2

Fractional portion = 0.125

UCBRSx = 0x10	User'sGuide Table 21-4:

Code:		EUSCI_A0->MCTLW= 0x1000 | UCOS16 | 0x0020;

EUSCI_A0->BRW = 78;

UCAxRXBUF Register ……eUSCI_Ax Receive Buffer Register

Figure 22-17. UCAxRXBUF Register

UCAxTXBUF Register …. eUSCI_Ax Transmit Buffer Register

Figure 22-18. UCAxTXBUF Register

UCAxIFG Register …. eUSCI_Ax Interrupt Flag Register

Figure 22-22. UCAxIFG Register

while  (   (EUSCI_A0 ->  IFG&0x01) == 0);		// wait till receiving end

while (   (EUSCI_A0 ->  IFG&0x02) == 0);		// wait till transmitting end

UCAxIE Register …. EUSCI_Ax Interrupt Enable Register

Figure 22-21. UCAxIE Register

EUSCI_A0->IE &= ~0x000F;    	 // disable interrupts

UCAxIV Register …. eUSCI_Ax Interrupt Vector Register

Figure 22-23. UCAxIV Register

include "msp.h"

void UART_Init(void){

EUSCI_A0->CTLW0 = 0x0001;  			// hold the USCI module in reset mode

// bit15=0,     		 no parity bits

// bit14=x,     		 not used when parity is disabled

// bit13=0,     		 LSB first

// bit12=0,     		 8-bit data length

// bit11=0,      		1 stop bit

// bits10-8=000, 	asynchronous UART mode

// bits7-6=11,  		clock source to SMCLK

// bit5=0,      		reject erroneous characters and do not set flag

// bit4=0,       		do not set flag for break characters

// bit3=0,      		not dormant

// bit2=0,       		transmit data, not address (not used here)

// bit1=0,       		do not transmit break (not used here)

// bit0=1,       		hold logic in reset state while configuring

EUSCI_A0->CTLW0 = 0x00C1;

EUSCI_A0->BRW = 26;          		// UCBR = baud rate = int (N) =26 = clock/baud rate = 26.0417

EUSCI_A0->MCTLW &= ~0xFFF1;  	// clear first and second modulation stage bit fields,

P1SEL0 |= 0x0C;

P1SEL1 &= ~0x0C;             			// configure P1.3 and P1.2 as primary module function

EUSCI_A0->CTLW0 &= ~0x0001;  		// enable the USCI module

EUSCI_A0->IE &= ~0x000F;    		// disable interrupts

}

// Wait for new serial port input, then return byte in receive buffer

char UART_InChar(void){

while ( (EUSCI_A0->IFG&0x01) == 0);

return ( (char)(EUSCI_A0->RXBUF));

}

// Wait for last byte to be sent, then output character by placing in transmit buffer

void UART_OutChar(char data){

while((EUSCI_A0->IFG&0x02) == 0);

EUSCI_A0->TXBUF = data;

}

volatile uint8_t inChar;

void main(void)

{

WDT_A->CTL = WDT_A_CTL_PW | WDT_A_CTL_HOLD;		// stop watchdog timer

UART_Init ( ); 				// initialize serial port for communication at 115200

while(1) {

inChar = UART_InChar();			 // wait for character from serial port keyboard emulator

UART_OutChar(inChar);			 // echo character back to serial port keyboard emulator

}

}